- GRE VPN

- NAT Overload

- OSPF

- ACL cho NAT
- ACL cho VLAN

- Inter-VLAN

- HSRP

Contents

Xây dựng nền tảng VLAN(tại mỗi chi nhánh)	4

1.	Tạo các VLAN và Switching(Layer 2)	4

* Chi nhánh BD	4

Trụ sở HCM	6

Chi nhánh DN	6

Chi nhánh CT	6

2.	SVI + HSRP	7

Chi nhánh BD	7

Trụ sở HCM	12

Chi nhánh Đồng Nai	18

Chi nhánh Cần Thơ	24

3.	Cấu hình DHCP	29

Chi nhánh BD	29

Trụ sở chính TP.HCM (HQ)	31

Chi nhánh Đồng Nai (DN)	32

Chi nhánh Cần Thơ (CT)	34

Cấu hình WLAN ở các WLC	35

Kết nối các chi nhánh(WAN)	36

4.	Cấu hình GRE VPN	36

5.	Cấu hình OSPF	53

1. Cấu hình OSPF cho 8 Switch Layer 3 (MLS)	53

2. Cấu hình OSPF cho 8 Router	58

III. Bảo mật và Truy cập Internet	62

6.	Cấu hình ACL	62

7.	Cấu hình NAT Overload(trên router)	70

8.	Yêu cầu "Y tế từ xa (Telemedicine)"	72

9.	Hỗ trợ truy cập từ xa với MFA	79

10.	Cấu hình Cloud & AI Ingretation	81

# Xây dựng nền tảng VLAN(tại mỗi chi nhánh)

## Tạo các VLAN và Switching(Layer 2)

### * Chi nhánh BD

- Tạo VLAN trên các switch (BD-SW1, BD-SW2, BD-SW3, BD-MLS1, BD-MLS2)

vlan 10

name BD_DOCTORS

vlan 20

name BD_NURSES

vlan 30

name BD_STAFF

vlan 50

name BD_STAFF_WIFI

vlan 60

name BD_GUEST_WIFI

vlan 99

name BD_MANAGEMENT

- Assign switchport to VLAN(trên switch)

+ Cấu hình trước trên BD-SW1, BD-SW2, BD-SW3, BD-SW-MGMT, còn BD-Staff và BD-Guest

. Trên BD-SW-MGMT

int range fa0/1-2

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,50,60,99

int range fa0/3-24

switchport mode access

switchport access vlan 99

interface Vlan99

ip address 10.20.99.11 255.255.255.192

ip default-gateway 10.20.99.62

. Trên BD-SW1

int range fa0/1-2

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99

int range fa0/3-24

switchport mode access

switchport access vlan 10

interface Vlan10

ip address 10.20.10.11 255.255.255.0

ip default-gateway 10.20.10.254

. Trên BD-SW2

int range fa0/1-2

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 20,99

int range fa0/3-24

switchport mode access

switchport access vlan 20

interface Vlan20

ip address 10.20.20.11 255.255.254.0

ip default-gateway 10.20.21.254

. Trên BD-SW3

int range fa0/1-2

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 30,99

int range fa0/3-24

switchport mode access

switchport access vlan 30

interface Vlan30

ip address 10.20.30.11 255.255.255.128

ip default-gateway 10.20.30.126

-> Tương tự với các switch ở các chi nhánh

. Trên HCM-SW4

int range fa0/1-2

switchport trunk native vlan 99

switchport mode trunk

int range fa0/3-24

switchport mode access

switchport access vlan 40

- Enable routing on Multilayer Switch(ip routing)

- Chọn mode: switchport trunk encapsulation dot1q(switch layer 3)

### Trụ sở HCM

vlan 10

name DOCTORS

vlan 20

name NURSES

vlan 30

name STAFF

vlan 40

name SERVERS

vlan 50

name STAFF_WIFI

vlan 60

name GUEST_WIFI

vlan 99

name MANAGEMENT

### Chi nhánh DN

vlan 10

name DN_DOCTORS

vlan 20

name DN_NURSES

vlan 30

name DN_STAFF

vlan 50

name DN_STAFF_WIFI

vlan 60

name DN_GUEST_WIFI

vlan 99

name DN_MANAGEMENT

### Chi nhánh CT

vlan 10

name CT_DOCTORS

vlan 20z

name CT_NURSES

vlan 30

name CT_STAFF

vlan 50

name CT_STAFF_WIFI

vlan 60

name CT_GUEST_WIFI

vlan 99

name CT_MANAGEMENT

## SVI + HSRP

### Chi nhánh BD

+ BD-MLS1

! ----- VLAN 10 (Doctor) -----

interface Vlan10

description BD_DOCTORS

ip address 10.20.10.1 255.255.255.0

standby version 2

standby 10 ip 10.20.10.254

standby 10 priority 150

standby 10 preempt

! ----- VLAN 20 (Nurses) -----

interface Vlan20

description BD_NURSES

ip address 10.20.20.1 255.255.254.0

standby version 2

standby 20 ip 10.20.21.254

standby 20 priority 150

standby 20 preempt

! ----- VLAN 30 (Staff) -----

interface Vlan30

description BD_STAFF

ip address 10.20.30.1 255.255.255.128

standby version 2

standby 30 ip 10.20.30.126

standby 30 priority 150

standby 30 preempt

! ----- VLAN 50 (Wifi-Staff) -----

interface Vlan50

description BD_STAFF_WIFI

ip address 10.20.50.1 255.255.254.0

standby version 2

standby 50 ip 10.20.51.254

standby 50 priority 150

standby 50 preempt

! ----- VLAN 60 (Guest) -----

interface Vlan60

description BD_GUEST_WIFI

ip address 10.20.60.1 255.255.254.0

standby version 2

standby 60 ip 10.20.61.254

standby 60 priority 150

standby 60 preempt

! ----- VLAN 99 (Management) -----

interface Vlan99

description MANAGEMENT

ip address 10.20.99.1 255.255.255.192

standby version 2

standby 99 ip 10.20.99.62

standby 99 priority 150

standby 99 preempt

----------------------------------

+ BD-MLS2

! ----- VLAN 10 (Doctor) -----

interface Vlan10

description BD_DOCTORS

ip address 10.20.10.2 255.255.255.0

standby version 2

standby 10 ip 10.20.10.254

standby 10 preempt

! ----- VLAN 20 (Nurses) -----

interface Vlan20

description BD_NURSES

ip address 10.20.20.2 255.255.254.0

standby version 2

standby 20 ip 10.20.21.254

standby 20 preempt

! ----- VLAN 30 (Staff) -----

interface Vlan30

description BD_STAFF

ip address 10.20.30.2 255.255.255.128

standby version 2

standby 30 ip 10.20.30.126

standby 30 preempt

! ----- VLAN 50 (Wifi-Staff) -----

interface Vlan50

description BD_STAFF_WIFI

ip address 10.20.50.2 255.255.254.0

standby version 2

standby 50 ip 10.20.51.254

standby 50 preempt

! ----- VLAN 60 (Guest) -----

interface Vlan60

description BD_GUEST_WIFI

ip address 10.20.60.2 255.255.254.0

standby version 2

standby 60 ip 10.20.61.254

standby 60 preempt

! ----- VLAN 99 (Management) -----

interface Vlan99

description MANAGEMENT

ip address 10.20.99.2 255.255.255.192

standby version 2

standby 99 ip 10.20.99.62

standby 99 preempt

- Mode trunk allowed/native

+ BD-MLS1

! ----- 0. Cổng nối xuống BD-SW-MGMT -----

interface fa0/5

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,50,60,99

! ----- 1. Cổng nối xuống BD-SW1 (chỉ cần VLAN 10 và 99) -----

interface fa0/6

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99

! ----- 2. Cổng nối xuống BD-SW2 (chỉ cần VLAN 20 và 99) -----

interface fa0/7

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 20,99

! ----- 3. Cổng nối xuống BD-SW3 (chỉ cần VLAN 30 và 99) -----

interface fa0/8

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 30,99

! ----- 4. Cổng nối WLC (cần VLAN 50, 60 và 99) -----

interface fa0/3

switchport mode access

switchport access vlan 99

switchport trunk encapsulation dot1q

no shutdown

### Trụ sở HCM

### 1. Cấu hình HCM-MLS1 (Active)

conf t

! ----- VLAN 10 (Doctors) /23 -----

interface Vlan10

description DOCTORS

ip address 10.10.10.1 255.255.254.0

standby version 2

standby 10 ip 10.10.11.254

standby 10 priority 120

standby 10 preempt

! ----- VLAN 20 (Nurses) /22 -----

interface Vlan20

description NURSES

ip address 10.10.20.1 255.255.252.0

standby version 2

standby 20 ip 10.10.23.254

standby 20 priority 120

standby 20 preempt

! ----- VLAN 30 (Staff) /24 -----

interface Vlan30

description STAFF

ip address 10.10.30.1 255.255.255.0

standby version 2

standby 30 ip 10.10.30.254

standby 30 priority 120

standby 30 preempt

! ----- VLAN 40 (Servers) /26 -----

interface Vlan40

description SERVERS

ip address 10.10.40.1 255.255.255.192

standby version 2

standby 40 ip 10.10.40.62

standby 40 priority 120

standby 40 preempt

! ----- VLAN 50 (Staff Wifi) /22 -----

interface Vlan50

description STAFF_WIFI

ip address 10.10.50.1 255.255.252.0

standby version 2

standby 50 ip 10.10.53.254

standby 50 priority 120

standby 50 preempt

! ----- VLAN 60 (Guest Wifi) /22 -----

interface Vlan60

description GUEST_WIFI

ip address 10.10.60.1 255.255.252.0

standby version 2

standby 60 ip 10.10.63.254

standby 60 priority 120

standby 60 preempt

! ----- VLAN 99 (Management) /26 -----

interface Vlan99

description MANAGEMENT

ip address 10.10.99.1 255.255.255.192

standby version 2

standby 99 ip 10.10.99.62

standby 99 priority 120

standby 99 preempt

### 2. Cấu hình HCM-MLS2 (Standby)

config terminal

! ----- VLAN 10 (Doctors) /23 -----

interface Vlan10

description DOCTORS

ip address 10.10.10.2 255.255.254.0

standby version 2

standby 10 ip 10.10.11.254

standby 10 preempt

! ----- VLAN 20 (Nurses) /22 -----

interface Vlan20

description NURSES

ip address 10.10.20.2 255.255.252.0

standby version 2

standby 20 ip 10.10.23.254

standby 20 preempt

! ----- VLAN 30 (Staff) /24 -----

interface Vlan30

description STAFF

ip address 10.10.30.2 255.255.255.0

standby version 2

standby 30 ip 10.10.30.254

standby 30 preempt

! ----- VLAN 40 (Servers) /26 -----

interface Vlan40

description SERVERS

ip address 10.10.40.2 255.255.255.192

standby version 2

standby 40 ip 10.10.40.62

standby 40 preempt

! ----- VLAN 50 (Staff Wifi) /22 -----

interface Vlan50

description STAFF_WIFI

ip address 10.10.50.2 255.255.252.0

standby version 2

standby 50 ip 10.10.53.254

standby 50 preempt

! ----- VLAN 60 (Guest Wifi) /22 -----

interface Vlan60

description GUEST_WIFI

ip address 10.10.60.2 255.255.252.0

standby version 2

standby 60 ip 10.10.63.254

standby 60 preempt

! ----- VLAN 99 (Management) /26 -----

interface Vlan99

description MANAGEMENT

ip address 10.10.99.2 255.255.255.192

standby version 2

standby 99 ip 10.10.99.62

standby 99 preempt

- Mode trunk allowed/native

+ HCM-MLS1 va HCM-MLS2

conf t

!

! ----- 1. Cổng nối xuống HCM-SW1 (VLAN 10 Doctors & 99) -----

interface FastEthernet0/6

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99

!

! ----- 2. Cổng nối xuống HCM-SW2 (VLAN 20 Nurses & 99) -----

interface FastEthernet0/7

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 20,99

!

! ----- 3. Cổng nối xuống HCM-SW3 (VLAN 30 Staff & 99) -----

interface FastEthernet0/8

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 30,99

!

! ----- 4. Cổng nối xuống HCM-SW4 (VLAN 40 Servers & 99) -----

! (Đây là switch riêng cho Data Center)

interface FastEthernet0/9

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 40,99

!

! ----- 5. Cổng nối WLC (HCM-WLC - VLAN 50 & 99) -----

interface fa0/3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 50,99

!

! ----- 6. Cổng nối Guest AP (HCM-Guest - VLAN 60 & 99) -----

interface fa0/4

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 60,99

!

end

### Chi nhánh Đồng Nai

### 1. Cấu hình DN-MLS1 (Active)

config terminal

! ----- VLAN 10 (Doctors) /24 -----

interface Vlan10

description DN_DOCTORS

ip address 10.30.10.1 255.255.255.0

standby version 2

standby 10 ip 10.30.10.254

standby 10 priority 120

standby 10 preempt

! ----- VLAN 20 (Nurses) /23 -----

interface Vlan20

description DN_NURSES

ip address 10.30.20.1 255.255.254.0

standby version 2

standby 20 ip 10.30.21.254

standby 20 priority 120

standby 20 preempt

! ----- VLAN 30 (Staff) /25 -----

interface Vlan30

description DN_STAFF

ip address 10.30.30.1 255.255.255.128

standby version 2

standby 30 ip 10.30.30.126

standby 30 priority 120

standby 30 preempt

! ----- VLAN 50 (Staff Wifi) /23 -----

interface Vlan50

description DN_STAFF_WIFI

ip address 10.30.50.1 255.255.254.0

standby version 2

standby 50 ip 10.30.51.254

standby 50 priority 120

standby 50 preempt

! ----- VLAN 60 (Guest Wifi) /23 -----

interface Vlan60

description DN_GUEST_WIFI

ip address 10.30.60.1 255.255.254.0

standby version 2

standby 60 ip 10.30.61.254

standby 60 priority 120

standby 60 preempt

! ----- VLAN 99 (Management) /26 -----

interface Vlan99

description DN_MANAGEMENT

ip address 10.30.99.1 255.255.255.192

standby version 2

standby 99 ip 10.30.99.62

standby 99 priority 120

standby 99 preempt

### 2. Cấu hình DN-MLS2 (Standby)

config terminal

! ----- VLAN 10 (Doctors) /24 -----

interface Vlan10

description DN_DOCTORS

ip address 10.30.10.2 255.255.255.0

standby version 2

standby 10 ip 10.30.10.254

standby 10 preempt

! ----- VLAN 20 (Nurses) /23 -----

interface Vlan20

description DN_NURSES

ip address 10.30.20.2 255.255.254.0

standby version 2

standby 20 ip 10.30.21.254

standby 20 preempt

! ----- VLAN 30 (Staff) /25 -----

interface Vlan30

description DN_STAFF

ip address 10.30.30.2 255.255.255.128

standby version 2

standby 30 ip 10.30.30.126

standby 30 preempt

! ----- VLAN 50 (Staff Wifi) /23 -----

interface Vlan50

description DN_STAFF_WIFI

ip address 10.30.50.2 255.255.254.0

standby version 2

standby 50 ip 10.30.51.254

standby 50 preempt

! ----- VLAN 60 (Guest Wifi) /23 -----

interface Vlan60

description DN_GUEST_WIFI

ip address 10.30.60.2 255.255.254.0

standby version 2

standby 60 ip 10.30.61.254

standby 60 preempt

! ----- VLAN 99 (Management) /26 -----

interface Vlan99

description DN_MANAGEMENT

ip address 10.30.99.2 255.255.255.192

standby version 2

standby 99 ip 10.30.99.62

standby 99 preempt

- Mode trunk allowed/native

config terminal

!

! ----- 1. Cổng nối xuống DN-SW1 (VLAN 10 & 99) -----

interface FastEthernet0/6

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99

!

! ----- 2. Cổng nối xuống DN-SW2 (VLAN 20 & 99) -----

interface FastEthernet0/7

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 20,99

!

! ----- 3. Cổng nối xuống DN-SW3 (VLAN 30 & 99) -----

interface FastEthernet0/8

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 30,99

!

! ----- 4. Cổng nối WLC (DN-STAFF-WIFI - VLAN 50 & 99) -----

interface fa0/3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 50,99

!

! ----- 5. Cổng nối Guest AP (DN-GUEST-WIFI - VLAN 60 & 99) -----

interface fa0/4

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 60,99

!

end

### Chi nhánh Cần Thơ

### 1. Cấu hình CT-MLS1 (Active)

config terminal

! ----- VLAN 10 (Doctors) /24 -----

interface Vlan10

description CT_DOCTORS

ip address 10.40.10.1 255.255.255.0

standby version 2

standby 10 ip 10.40.10.254

standby 10 priority 120

standby 10 preempt

! ----- VLAN 20 (Nurses) /23 -----

interface Vlan20

description CT_NURSES

ip address 10.40.20.1 255.255.254.0

standby version 2

standby 20 ip 10.40.21.254

standby 20 priority 120

standby 20 preempt

! ----- VLAN 30 (Staff) /25 -----

interface Vlan30

description CT_STAFF

ip address 10.40.30.1 255.255.255.128

standby version 2

standby 30 ip 10.40.30.126

standby 30 priority 120

standby 30 preempt

! ----- VLAN 50 (Staff Wifi) /23 -----

interface Vlan50

description CT_STAFF_WIFI

ip address 10.40.50.1 255.255.254.0

standby version 2

standby 50 ip 10.40.51.254

standby 50 priority 120

standby 50 preempt

! ----- VLAN 60 (Guest Wifi) /23 -----

interface Vlan60

description CT_GUEST_WIFI

ip address 10.40.60.1 255.255.254.0

standby version 2

standby 60 ip 10.40.61.254

standby 60 priority 120

standby 60 preempt

! ----- VLAN 99 (Management) /26 -----

interface Vlan99

description CT_MANAGEMENT

ip address 10.40.99.1 255.255.255.192

standby version 2

standby 99 ip 10.40.99.62

standby 99 priority 120

standby 99 preempt

### 2. Cấu hình CT-MLS2 (Standby)

config terminal

! ----- VLAN 10 (Doctors) /24 -----

interface Vlan10

description CT_DOCTORS

ip address 10.40.10.2 255.255.255.0

standby version 2

standby 10 ip 10.40.10.254

standby 10 preempt

! ----- VLAN 20 (Nurses) /23 -----

interface Vlan20

description CT_NURSES

ip address 10.40.20.2 255.255.254.0

standby version 2

standby 20 ip 10.40.21.254

standby 20 preempt

! ----- VLAN 30 (Staff) /25 -----

interface Vlan30

description CT_STAFF

ip address 10.40.30.2 255.255.255.128

standby version 2

standby 30 ip 10.40.30.126

standby 30 preempt

! ----- VLAN 50 (Staff Wifi) /23 -----

interface Vlan50

description CT_STAFF_WIFI

ip address 10.40.50.2 255.255.254.0

standby version 2

standby 50 ip 10.40.51.254

standby 50 preempt

! ----- VLAN 60 (Guest Wifi) /23 -----

interface Vlan60

description CT_GUEST_WIFI

ip address 10.40.60.2 255.255.254.0

standby version 2

standby 60 ip 10.40.61.254

standby 60 preempt

! ----- VLAN 99 (Management) /26 -----

interface Vlan99

description CT_MANAGEMENT

ip address 10.40.99.2 255.255.255.192

standby version 2

standby 99 ip 10.40.99.62

standby 99 preempt

- Mode trunk allowed/native

config terminal

!

! ----- 1. Cổng nối xuống CT-SW1 (VLAN 10 & 99) -----

interface FastEthernet0/6

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99

!

! ----- 2. Cổng nối xuống CT-SW2 (VLAN 20 & 99) -----

interface FastEthernet0/7

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 20,99

!

! ----- 3. Cổng nối xuống CT-SW3 (VLAN 30 & 99) -----

interface FastEthernet0/8

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 30,99

!

! ----- 4. Cổng nối WLC (CT-WLC - VLAN 50 & 99) -----

Interface fa0/3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 50,99

!

! ----- 5. Cổng nối Guest AP (CT-Guest - VLAN 60 & 99) -----

interface fa0/4

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 60,99

!

end

## Cấu hình DHCP

### Chi nhánh BD

- Trên BD-MLS1 và BD-MLS2

config terminal

! ----- 1. LOẠI TRỪ IP TĨNH (EXCLUDED ADDRESS) -----

! V10: BD_DOCTORS (10.20.10.0/24) -> Exclude .1-.19 & .251-.254

ip dhcp excluded-address 10.20.10.1 10.20.10.19

ip dhcp excluded-address 10.20.10.251 10.20.10.254

! V20: BD_NURSES (10.20.20.0/23) -> Exclude .1-.9 & .251-.254 (của subnet .21)

ip dhcp excluded-address 10.20.20.1 10.20.20.9

ip dhcp excluded-address 10.20.21.251 10.20.21.254

! V30: BD_STAFF (10.20.30.0/25) -> Exclude .1-.9 & .121-.126

ip dhcp excluded-address 10.20.30.1 10.20.30.9

ip dhcp excluded-address 10.20.30.121 10.20.30.126

! V50: BD_WIFI_STAFF (10.20.50.0/23) -> Exclude .1-.9 & .251-.254 (của subnet .51)

ip dhcp excluded-address 10.20.50.1 10.20.50.9

ip dhcp excluded-address 10.20.51.251 10.20.51.254

! V60: BD_GUEST (10.20.60.0/23) -> Exclude .1-.9 & .251-.254 (của subnet .61)

ip dhcp excluded-address 10.20.60.1 10.20.60.9

ip dhcp excluded-address 10.20.61.251 10.20.61.254

! ----- 2. CẤU HÌNH DHCP POOLS -----

ip dhcp pool BD_DOCTORS_POOL

network 10.20.10.0 255.255.255.0

default-router 10.20.10.254

dns-server 8.8.8.8

ip dhcp pool BD_NURSES_POOL

network 10.20.20.0 255.255.254.0

default-router 10.20.21.254

dns-server 8.8.8.8

ip dhcp pool BD_STAFF_POOL

network 10.20.30.0 255.255.255.128

default-router 10.20.30.126

dns-server 8.8.8.8

ip dhcp pool BD_WIFI_STAFF_POOL

network 10.20.50.0 255.255.254.0

default-router 10.20.51.254

dns-server 8.8.8.8

ip dhcp pool BD_GUEST_POOL

network 10.20.60.0 255.255.254.0

default-router 10.20.61.254

dns-server 8.8.8.8

### Trụ sở chính TP.HCM (HQ)

Áp dụng trên HCM-MLS1 và HCM-MLS2

config terminal

! ----- EXCLUDED ADDRESS -----

! V10 Doctors

ip dhcp excluded-address 10.10.10.1 10.10.10.49

ip dhcp excluded-address 10.10.11.251 10.10.11.254

! V20 Nurses

ip dhcp excluded-address 10.10.20.1 10.10.20.49

ip dhcp excluded-address 10.10.23.251 10.10.23.254

! V30 Staff

ip dhcp excluded-address 10.10.30.1 10.10.30.19

ip dhcp excluded-address 10.10.30.251 10.10.30.254

! V50 Wifi Staff

ip dhcp excluded-address 10.10.50.1 10.10.50.9

ip dhcp excluded-address 10.10.53.251 10.10.53.254

! V60 Guest Wifi

ip dhcp excluded-address 10.10.60.1 10.10.60.1

ip dhcp excluded-address 10.10.63.251 10.10.63.254

! ----- DHCP POOLS -----

ip dhcp pool DOCTORS_POOL

network 10.10.10.0 255.255.254.0

default-router 10.10.11.254

dns-server 8.8.8.8

ip dhcp pool NURSES_POOL

network 10.10.20.0 255.255.252.0

default-router 10.10.23.254

dns-server 8.8.8.8

ip dhcp pool STAFF_POOL

network 10.10.30.0 255.255.255.0

default-router 10.10.30.254

dns-server 8.8.8.8

ip dhcp pool WIFI_STAFF_POOL

network 10.10.50.0 255.255.252.0

default-router 10.10.53.254

dns-server 8.8.8.8

ip dhcp pool WIFI_GUEST_POOL

network 10.10.60.0 255.255.252.0

default-router 10.10.63.254

dns-server 8.8.8.8

### Chi nhánh Đồng Nai (DN)

Áp dụng trên DN-MLS1 và DN-MLS2

config terminal

! ----- EXCLUDED ADDRESS -----

ip dhcp excluded-address 10.30.10.1 10.30.10.19

ip dhcp excluded-address 10.30.10.251 10.30.10.254

ip dhcp excluded-address 10.30.20.1 10.30.20.9

ip dhcp excluded-address 10.30.21.251 10.30.21.254

ip dhcp excluded-address 10.30.30.1 10.30.30.9

ip dhcp excluded-address 10.30.30.121 10.30.30.126

ip dhcp excluded-address 10.30.50.1 10.30.50.9

ip dhcp excluded-address 10.30.51.251 10.30.51.254

ip dhcp excluded-address 10.30.60.1 10.30.60.9

ip dhcp excluded-address 10.30.61.251 10.30.61.254

! ----- DHCP POOLS -----

ip dhcp pool DN_DOCTORS_POOL

network 10.30.10.0 255.255.255.0

default-router 10.30.10.254

dns-server 8.8.8.8

ip dhcp pool DN_NURSES_POOL

network 10.30.20.0 255.255.254.0

default-router 10.30.21.254

dns-server 8.8.8.8

ip dhcp pool DN_STAFF_POOL

network 10.30.30.0 255.255.255.128

default-router 10.30.30.126

dns-server 8.8.8.8

ip dhcp pool DN_WIFI_STAFF_POOL

network 10.30.50.0 255.255.254.0

default-router 10.30.51.254

dns-server 8.8.8.8

ip dhcp pool DN_GUEST_POOL

network 10.30.60.0 255.255.254.0

default-router 10.30.61.254

dns-server 8.8.8.8

### Chi nhánh Cần Thơ (CT)

Áp dụng trên CT-MLS1 và CT-MLS2

config terminal

! ----- EXCLUDED ADDRESS -----

ip dhcp excluded-address 10.40.10.1 10.40.10.19

ip dhcp excluded-address 10.40.10.251 10.40.10.254

ip dhcp excluded-address 10.40.20.1 10.40.20.9

ip dhcp excluded-address 10.40.21.251 10.40.21.254

ip dhcp excluded-address 10.40.30.1 10.40.30.9

ip dhcp excluded-address 10.40.30.121 10.40.30.126

ip dhcp excluded-address 10.40.50.1 10.40.50.9

ip dhcp excluded-address 10.40.51.251 10.40.51.254

ip dhcp excluded-address 10.40.60.1 10.40.60.9

ip dhcp excluded-address 10.40.61.251 10.40.61.254

! ----- DHCP POOLS -----

ip dhcp pool CT_DOCTORS_POOL

network 10.40.10.0 255.255.255.0

default-router 10.40.10.254

dns-server 8.8.8.8

ip dhcp pool CT_NURSES_POOL

network 10.40.20.0 255.255.254.0

default-router 10.40.21.254

dns-server 8.8.8.8

!

ip dhcp pool CT_STAFF_POOL

network 10.40.30.0 255.255.255.128

default-router 10.40.30.126

dns-server 8.8.8.8

!

ip dhcp pool CT_WIFI_STAFF_POOL

network 10.40.50.0 255.255.254.0

default-router 10.40.51.254

dns-server 8.8.8.8

!

ip dhcp pool CT_GUEST_POOL

network 10.40.60.0 255.255.254.0

default-router 10.40.61.254

dns-server 8.8.8.8

!

# Cấu hình WLAN ở các WLC

- có thể dùng 1wlc để tạo ra 2 ssid

Cấu hình Phương thức Đăng nhập (Security)

Đây là lúc bạn tạo 2 SSID và áp dụng "Captive Portal" cho khách.

- Tạo WLAN (SSID) cho Nhân viên (Private):

  - Vào WLANs > Create New.

  - SSID Name: MedCare-NhanVien (hoặc tên tương tự).

  - Interface: Chọn staff-wifi (VLAN 50) mà bạn vừa tạo.

  - Security (Tab): Chọn WPA2 + WPA2 và WPA2-PSK (hoặc 802.1x nếu có máy chủ RADIUS). Đặt một mật khẩu mạnh.

  - Kết quả: Đây là hình thức "đăng nhập" của nhân viên (dùng mật khẩu Wi-Fi).

- Tạo WLAN (SSID) cho Bệnh nhân (Public) với Captive Portal:

  - Vào WLANs > Create New.

  - SSID Name: MedCare-Guest (hoặc tên tương tự).

  - Interface: Chọn guest-wifi (VLAN 60) mà bạn vừa tạo.

  - Security (Tab):

    - Layer 2: Chọn None (để mạng mở, không cần mật khẩu Wi-Fi).

    - Layer 3: Tích vào ô Web Policy và chọn We	b Authentication.

  - Kết quả: Đây chính là Captive Portal. Khi bệnh nhân kết nối, họ sẽ tự động bị chuyển hướng đến một trang web (portal) để nhấn "Đồng ý" trước khi được truy cập Internet.

# Kết nối các chi nhánh(WAN)

## Cấu hình GRE VPN

- Cần cấu hình thêm IP giữa ISP và các router để làm vpn

+ Cấu hình IP

enable config terminal ! ----- 1. Nối đến HQ1 ----- interface Serial0/1/0 ip address 100.0.0.1 255.255.255.252 clock rate 64000 no shutdown ! ----- 2. Nối đến HQ2 ----- interface Serial0/1/1 ip address 100.0.0.5 255.255.255.252 clock rate 64000 no shutdown ! ----- 3. Nối đến BD1 ----- interface Serial0/0/0 ip address 100.0.0.9 255.255.255.252 clock rate 64000 no shutdown ! ----- 4. Nối đến BD2 ----- interface Serial0/0/1 ip address 100.0.0.13 255.255.255.252 clock rate 64000 no shutdown ! ----- 5. Nối đến DN1 ----- interface Serial0/2/0 ip address 100.0.0.17 255.255.255.252 clock rate 64000 no shutdown ! ----- 6. Nối đến DN2 ----- interface Serial0/2/1 ip address 100.0.0.21 255.255.255.252 clock rate 64000 no shutdown ! ----- 7. Nối đến CT1 ----- interface Serial0/3/0 ip address 100.0.0.25 255.255.255.252 clock rate 64000 no shutdown ! ----- 8. Nối đến CT2 ----- interface Serial0/3/1 ip address 100.0.0.29 255.255.255.252 clock rate 64000 no shutdown end

! ----- Trên Router HQ1 ----- enable config terminal hostname HQ1 interface Serial0/3/0 ip address 100.0.0.2 255.255.255.252 no shutdown

! ----- Trên Router HQ2 ----- enable config terminal hostname HQ2 ! interface Serial0/3/0 ip address 100.0.0.6 255.255.255.252 no shutdown !

! ----- Trên Router Branch-BD1 ----- enable config terminal hostname Branch-BD1 ! interface Serial0/3/0 ip address 100.0.0.10 255.255.255.252 no shutdown

! ----- Trên Router Branch-BD2 ----- enable config terminal hostname Branch-BD2 interface Serial0/3/0 ip address 100.0.0.14 255.255.255.252 no shutdown

! ----- Trên Router Branch-DN1 ----- enable config terminal hostname Branch-DN1 ! interface Serial0/3/0 ip address 100.0.0.18 255.255.255.252 no shutdown !

! ----- Trên Router Branch-DN2 ----- enable config terminal hostname Branch-DN2 ! interface Serial0/3/0 ip address 100.0.0.22 255.255.255.252 no shutdown !

! ----- Trên Router Branch-CT1 ----- enable config terminal hostname Branch-CT1 ! interface Serial0/3/0 ip address 100.0.0.26 255.255.255.252 no shutdown !

! ----- Trên Router Branch-CT2 ----- enable config terminal hostname Branch-CT2 interface Serial0/3/0 ip address 100.0.0.30 255.255.255.252 no shutdown

+ VPN

Table 1

---A(HQ1) interface Tunnel1 tunnel mode gre ip ip address 172.25.0.1 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.10 ---B(Branch-BD1) interface Tunnel1 tunnel mode gre ip ip address 172.25.0.2 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.2

---A(HQ1) interface Tunnel2 tunnel mode gre ip ip address 172.25.0.5 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.14 ---B(Branch-BD2) interface Tunnel2 tunnel mode gre ip ip address 172.25.0.6 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.2

Table 2

---A(HQ2) interface Tunnel3 tunnel mode gre ip ip address 172.25.0.9 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.10 ---B(Branch-BD1) interface Tunnel3 tunnel mode gre ip ip address 172.25.0.10 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.6

Table 3

---A(HQ2) interface Tunnel4 tunnel mode gre ip ip address 172.25.0.13 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.14 ---B(Branch-BD2) interface Tunnel4 tunnel mode gre ip ip address 172.25.0.14 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.6

Table 4

---A(HQ1) interface Tunnel5 tunnel mode gre ip ip address 172.25.1.1 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.18 ---B(Branch-DN1) interface Tunnel5 tunnel mode gre ip ip address 172.25.1.2 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.2

---A(HQ1) interface Tunnel6 tunnel mode gre ip ip address 172.25.1.5 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.22 ---B(Branch-DN2) interface Tunnel6 tunnel mode gre ip ip address 172.25.1.6 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.2

Table 5

---A(HQ2) interface Tunnel7 tunnel mode gre ip ip address 172.25.1.9 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.18 ---B(Branch-DN1) interface Tunnel7 tunnel mode gre ip ip address 172.25.1.10 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.6

Table 6

---A(HQ2) interface Tunnel8 tunnel mode gre ip ip address 172.25.1.13 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.22 ---B(Branch-DN2) interface Tunnel8 tunnel mode gre ip ip address 172.25.1.14 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.6

Table 7

---A(HQ1) interface Tunnel9 tunnel mode gre ip ip address 172.25.2.1 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.26 ---B(Branch-CT1) interface Tunnel9 tunnel mode gre ip ip address 172.25.2.2 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.2

---A(HQ1) interface Tunnel10 tunnel mode gre ip ip address 172.25.2.5 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.30 ---B(Branch-CT2) interface Tunnel10 tunnel mode gre ip ip address 172.25.2.6 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.2

Table 8

---A(HQ2) interface Tunnel11 tunnel mode gre ip ip address 172.25.2.9 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.26 ---B(Branch-CT1) interface Tunnel11 tunnel mode gre ip ip address 172.25.2.10 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.6

Table 9

---A(HQ2) interface Tunnel12 tunnel mode gre ip ip address 172.25.2.13 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.30 ---B(Branch-CT2) interface Tunnel12 tunnel mode gre ip ip address 172.25.2.14 255.255.255.252 tunnel source se0/3/0 tunnel destination 100.0.0.6

2. Cấu hình OSPF

- Cấu hình ip

! ----- Trên Router HQ1 ----- enable config terminal interface Gi0/1 ip address 10.254.10.1 255.255.255.252 no shutdown interface Gi0/0 ip address 10.254.10.5 255.255.255.252 no shutdown

! ----- Trên Router HQ2 ----- enable config terminal interface Gi0/1 ip address 10.254.10.13 255.255.255.252 no shutdown interface Gi0/0 ip address 10.254.10.9 255.255.255.252 no shutdown

! ----- Trên Router Branch-BD1 ----- enable config terminal interface Gi0/1 ip address 10.254.20.1 255.255.255.252 no shutdown interface Gi0/0 ip address 10.254.20.5 255.255.255.252 no shutdown

! ----- Trên Router Branch-BD2 ----- enable config terminal interface Gi0/1 ip address 10.254.20.13 255.255.255.252 no shutdown interface Gi0/0 ip address 10.254.20.9 255.255.255.252 no shutdown

! ----- Trên Router Branch-DN1 ----- enable config terminal interface Gi0/1 ip address 10.254.30.1 255.255.255.252 no shutdown interface Gi0/0 ip address 10.254.30.5 255.255.255.252 no shutdown

! ----- Trên Router Branch-DN2 ----- enable config terminal interface Gi0/1 ip address 10.254.30.13 255.255.255.252 no shutdown interface Gi0/0 ip address 10.254.30.9 255.255.255.252 no shutdown

! ----- Trên Router Branch-CT1 ----- enable config terminal interface Gi0/1 ip address 10.254.40.1 255.255.255.252 no shutdown interface Gi0/0 ip address 10.254.40.5 255.255.255.252 no shutdown

! ----- Trên Router Branch-CT2 ----- enable config terminal interface Gi0/1 ip address 10.254.40.13 255.255.255.252 no shutdown interface Gi0/0 ip address 10.254.40.9 255.255.255.252 no shutdown

Table 10

! ----- Trên SWL3 BD-MLS1 ----- enable config terminal interface fa0/1 ip address 10.254.20.10 255.255.255.252 no shutdown interface fa0/2 ip address 10.254.20.2 255.255.255.252 no shutdown

Table 11

! ----- Trên SWL3 BD-MLS2 ----- enable config terminal interface fa0/1 ip address 10.254.20.6 255.255.255.252 no shutdown interface fa0/2 ip address 10.254.20.14 255.255.255.252 no shutdown

Table 12

! ----- Trên SWL3 HCM-MLS1 ----- enable config terminal interface fa0/1 ip address 10.254.10.10 255.255.255.252 no shutdown interface fa0/2 ip address 10.254.10.2 255.255.255.252 no shutdown

Table 13

! ----- Trên SWL3 HCM-MLS2 ----- enable config terminal interface fa0/1 ip address 10.254.10.6 255.255.255.252 no shutdown interface fa0/2 ip address 10.254.10.14 255.255.255.252 no shutdown

Table 14

! ----- Trên SWL3 DN-MLS1 ----- enable config terminal interface fa0/1 ip address 10.254.30.10 255.255.255.252 no shutdown interface fa0/2 ip address 10.254.30.2 255.255.255.252 no shutdown

! ----- Trên SWL3 DN-MLS2 ----- enable config terminal interface fa0/1 ip address 10.254.30.6 255.255.255.252 no shutdown interface fa0/2 ip address 10.254.30.14 255.255.255.252 no shutdown

Table 15

! ----- Trên SWL3 CT-MLS1 ----- enable config terminal interface fa0/1 ip address 10.254.40.10 255.255.255.252 no shutdown interface fa0/2 ip address 10.254.40.2 255.255.255.252 no shutdown

Table 16

! ----- Trên SWL3 CT-MLS2 ----- enable config terminal interface fa0/1 ip address 10.254.40.6 255.255.255.252 no shutdown interface fa0/2 ip address 10.254.40.14 255.255.255.252 no shutdown

## Cấu hình OSPF

### 1. Cấu hình OSPF cho 8 Switch Layer 3 (MLS)

Trụ sở chính: HCM-MLS1

config terminal

router ospf 1

router-id 10.10.99.1

passive-interface default

no passive-interface FastEthernet0/1

no passive-interface FastEthernet0/2

!

! Quảng bá mạng kết nối

network 10.254.10.0 0.0.0.3 area 0

network 10.254.10.8 0.0.0.3 area 0

!

! Quảng bá VLAN (Cập nhật theo bảng mới)

network 10.10.10.0 0.0.1.255 area 0

network 10.10.20.0 0.0.3.255 area 0

network 10.10.30.0 0.0.0.255 area 0

network 10.10.40.0 0.0.0.63 area 0

network 10.10.50.0 0.0.3.255 area 0

network 10.10.60.0 0.0.3.255 area 0

network 10.10.99.0 0.0.0.63 area 0

endTrụ sở chính: HCM-MLS2

config terminal

router ospf 1

router-id 10.10.99.2

passive-interface default

no passive-interface FastEthernet0/1

no passive-interface FastEthernet0/2

!

network 10.254.10.4 0.0.0.3 area 0

network 10.254.10.12 0.0.0.3 area 0

!

network 10.10.10.0 0.0.1.255 area 0

network 10.10.20.0 0.0.3.255 area 0

network 10.10.30.0 0.0.0.255 area 0

network 10.10.40.0 0.0.0.63 area 0

network 10.10.50.0 0.0.3.255 area 0

network 10.10.60.0 0.0.3.255 area 0

network 10.10.99.0 0.0.0.63 area 0

endChi nhánh Bình Dương: BD-MLS1

config terminal

router ospf 1

router-id 10.20.99.1

area 20 stub

passive-interface default

no passive-interface FastEthernet0/1

no passive-interface FastEthernet0/2

!

network 10.254.20.0 0.0.0.3 area 0

network 10.254.20.8 0.0.0.3 area 0

!

network 10.20.10.0 0.0.0.255 area 0

network 10.20.20.0 0.0.1.255 area 0

network 10.20.30.0 0.0.0.127 area 0

network 10.20.50.0 0.0.1.255 area 0

network 10.20.60.0 0.0.1.255 area 0

network 10.20.99.0 0.0.0.63 area 0

endChi nhánh Bình Dương: BD-MLS2

config terminal

router ospf 1

router-id 10.20.99.2

area 20 stub

passive-interface default

no passive-interface FastEthernet0/1

no passive-interface FastEthernet0/2

!

network 10.254.20.4 0.0.0.3 area 0

network 10.254.20.12 0.0.0.3 area 0

!

network 10.20.10.0 0.0.0.255 area 0

network 10.20.20.0 0.0.1.255 area 0

network 10.20.30.0 0.0.0.127 area 0

network 10.20.50.0 0.0.1.255 area 0

network 10.20.60.0 0.0.1.255 area 0

network 10.20.99.0 0.0.0.63 area 0

endChi nhánh Đồng Nai: DN-MLS1

config terminal

router ospf 1

router-id 10.30.99.1

passive-interface default

no passive-interface FastEthernet0/1

no passive-interface FastEthernet0/2

!

network 10.254.30.0 0.0.0.3 area 0

network 10.254.30.8 0.0.0.3 area 0

!

network 10.30.10.0 0.0.0.255 area 0

network 10.30.20.0 0.0.1.255 area 0

network 10.30.30.0 0.0.0.127 area 0

network 10.30.50.0 0.0.1.255 area 0

network 10.30.60.0 0.0.1.255 area 0

network 10.30.99.0 0.0.0.63 area 0

endChi nhánh Đồng Nai: DN-MLS2

config terminal

router ospf 1

router-id 10.30.99.2

passive-interface default

no passive-interface FastEthernet0/1

no passive-interface FastEthernet0/2

!

network 10.254.30.4 0.0.0.3 area 0

network 10.254.30.12 0.0.0.3 area 0

!

network 10.30.10.0 0.0.0.255 area 0

network 10.30.20.0 0.0.1.255 area 0

network 10.30.30.0 0.0.0.127 area 0

network 10.30.50.0 0.0.1.255 area 0

network 10.30.60.0 0.0.1.255 area 0

network 10.30.99.0 0.0.0.63 area 0

endChi nhánh Cần Thơ: CT-MLS1

config terminal

router ospf 1

router-id 10.40.99.1

passive-interface default

no passive-interface FastEthernet0/1

no passive-interface FastEthernet0/2

!

network 10.254.40.0 0.0.0.3 area 0

network 10.254.40.8 0.0.0.3 area 0

!

network 10.40.10.0 0.0.0.255 area 0

network 10.40.20.0 0.0.1.255 area 0

network 10.40.30.0 0.0.0.127 area 0

network 10.40.50.0 0.0.1.255 area 0

network 10.40.60.0 0.0.1.255 area 0

network 10.40.99.0 0.0.0.63 area 0

endChi nhánh Cần Thơ: CT-MLS2

config terminal

router ospf 1

router-id 10.40.99.2

passive-interface default

no passive-interface FastEthernet0/1

no passive-interface FastEthernet0/2

!

network 10.254.40.4 0.0.0.3 area 0

network 10.254.40.12 0.0.0.3 area 0

!

network 10.40.10.0 0.0.0.255 area 0

network 10.40.20.0 0.0.1.255 area 0

network 10.40.30.0 0.0.0.127 area 0

network 10.40.50.0 0.0.1.255 area 0

network 10.40.60.0 0.0.1.255 area 0

network 10.40.99.0 0.0.0.63 area 0

end

### 2. Cấu hình OSPF cho 8 Router

Router HQ1

config terminal

router ospf 1

! Quảng bá các mạng kết nối và Tunnel

network 10.254.10.0 0.0.0.3 area 0

network 10.254.10.4 0.0.0.3 area 0

network 172.25.0.0 0.0.0.3 area 0

network 172.25.0.4 0.0.0.3 area 0

network 172.25.1.0 0.0.0.3 area 0

network 172.25.1.4 0.0.0.3 area 0

network 172.25.2.0 0.0.0.3 area 0

network 172.25.2.4 0.0.0.3 area 0           !

Router HQ2

config terminal

router ospf 1

! Quảng bá các mạng

network 10.254.10.8 0.0.0.3 area 0

network 10.254.10.12 0.0.0.3 area 0

network 172.25.0.8 0.0.0.3 area 0

network 172.25.0.12 0.0.0.3 area 0

network 172.25.1.8 0.0.0.3 area 0

network 172.25.1.12 0.0.0.3 area 0

network 172.25.2.8 0.0.0.3 area 0

network 172.25.2.12 0.0.0.3 area 0          !

Router Branch-BD1

config terminal

router ospf 1

! Quảng bá các mạng

network 10.254.20.0 0.0.0.3 area 0

network 10.254.20.4 0.0.0.3 area 0

network 172.25.0.0 0.0.0.3 area 0

network 172.25.0.8 0.0.0.3 area 0

Router Branch-BD2

config terminal

router ospf 1

! Quảng bá các mạng

network 10.254.20.8 0.0.0.3 area 0

network 10.254.20.12 0.0.0.3 area 0

network 172.25.0.4 0.0.0.3 area 0

network 172.25.0.12 0.0.0.3 area 0

Router Branch-DN1

config terminal

router ospf 1

! Quảng bá các mạng

network 10.254.30.0 0.0.0.3 area 0

! Link to DN-MLS1

network 10.254.30.4 0.0.0.3 area 0

! Link to DN-MLS2

network 172.25.1.0 0.0.0.3 area 0

! Tunnel5 (HQ1)

network 172.25.1.8 0.0.0.3 area 0

! Tunnel7 (HQ2)

Router Branch-DN2

config terminal

router ospf 1

! Quảng bá các mạng

network 10.254.30.8 0.0.0.3 area 0

! Link to DN-MLS1 [cite: 31]

network 10.254.30.12 0.0.0.3 area 0

! Link to DN-MLS2 [cite: 31]

network 172.25.1.4 0.0.0.3 area 0

! Tunnel6 (HQ1) [cite: 33]

network 172.25.1.12 0.0.0.3 area 0

! Tunnel8 (HQ2) [cite: 33]

Router Branch-CT1

config terminal

router ospf 1

! Quảng bá các mạng

network 10.254.40.0 0.0.0.3 area 0

! Link to CT-MLS1

network 10.254.40.4 0.0.0.3 area 0

! Link to CT-MLS2

network 172.25.2.0 0.0.0.3 area 0

! Tunnel9 (HQ1)

network 172.25.2.8 0.0.0.3 area 0

! Tunnel11 (HQ2)

Router Branch-CT2

config terminal

router ospf 1

! Quảng bá các mạng

network 10.254.40.8 0.0.0.3 area 0

! Link to CT-MLS1

network 10.254.40.12 0.0.0.3 area 0

! Link to CT-MLS2

network 172.25.2.4 0.0.0.3 area 0

! Tunnel10 (HQ1)

network 172.25.2.12 0.0.0.3 area 0

! Tunnel12 (HQ2)

ISP(config)#router ospf 1

ISP(config-router)# network 100.0.0.0 0.0.0.3 area 0

ISP(config-router)# network 100.0.0.4 0.0.0.3 area 0

ISP(config-router)# network 100.0.0.8 0.0.0.3 area 0

ISP(config-router)# network 100.0.0.12 0.0.0.3 area 0

ISP(config-router)# network 100.0.0.16 0.0.0.3 area 0

ISP(config-router)# network 100.0.0.20 0.0.0.3 area 0

ISP(config-router)# network 100.0.0.24 0.0.0.3 area 0

ISP(config-router)# network 100.0.0.28 0.0.0.3 area 0

# III. Bảo mật và Truy cập Internet

## Cấu hình ACL

- 2 nhóm ACL chính

+ ACL bảo vệ Server (Quan trọng nhất): Một ACL chuyên dụng đặt tại Trụ sở chính (HQ) để chỉ cho phép các VLAN Y tế (Bác sĩ, Nhân viên) từ tất cả các chi nhánh được truy cập vào Server.

+ ACL Cô lập (Zero Trust): Các ACL đặt tại mỗi chi nhánh để ngăn các VLAN (như Khách, Nhân viên) truy cập vào các VLAN không liên quan, đặc biệt là VLAN Quản lý và VLAN Server.

* Trụ sở chính (HQ)

1. ACL Cơ bản (Standard) - Bảo vệ Server (VLAN 40)

Vị trí: Áp dụng trên interface Vlan40 theo chiều out (từ Switch đi ra Server). Nhiệm vụ: Chỉ cho phép IP của Bác sĩ/Y tá từ khắp mọi nơi đi vào Server.

Cisco CLI

config terminal

! Tạo Standard ACL (Số 1 đến 99)

access-list 40 remark ### CHO PHEP BAC SI VA Y TA TOAN HE THONG ###

!

! --- Trụ sở HQ ---

access-list 40 permit 10.10.10.0 0.0.1.255

access-list 40 permit 10.10.20.0 0.0.3.255

!

! --- Bình Dương ---

access-list 40 permit 10.20.10.0 0.0.0.255

access-list 40 permit 10.20.20.0 0.0.1.255

!

! --- Đồng Nai ---

access-list 40 permit 10.30.10.0 0.0.0.255

access-list 40 permit 10.30.20.0 0.0.1.255

!

! --- Cần Thơ ---

access-list 40 permit 10.40.10.0 0.0.0.255

access-list 40 permit 10.40.20.0 0.0.1.255

! (Mặc định có deny any ở cuối)

!

! Áp dụng vào cổng Server (chiều đi ra khỏi router vào server)

interface Vlan40

ip access-group 40 out

exit

2. ACL Mở rộng (Extended) - Cô lập người dùng HQ

Vị trí: Áp dụng trên interface Vlan10, 20, 30... theo chiều in.

Cisco CLI

! --- 1. ACL cho Bác sĩ & Y tá HQ (Được vào Server, Cấm mạng khác) ---

ip access-list extended HQ_MEDICAL_IN

permit udp any any eq bootps

permit udp any any eq domain

! Cho phép đi đến Server HQ

permit ip any 10.10.16.128 0.0.0.63

! CHẶN truy cập tất cả mạng nội bộ (10.x.x.x) -> Chặn luôn các VLAN khác và Chi nhánh khác

deny   ip any 10.0.0.0 0.255.255.255

! Cho phép ra Internet

permit ip any any

!

interface Vlan10

ip access-group HQ_MEDICAL_IN in

interface Vlan20

ip access-group HQ_MEDICAL_IN in

! --- 2. ACL cho Nhân viên & Khách HQ (Cấm tất cả nội bộ) ---

ip access-list extended HQ_ISOLATE_IN

permit udp any any eq bootps

permit udp any any eq domain

! Chặn toàn bộ mạng nội bộ (Bao gồm cả Server)

deny   ip any 10.0.0.0 0.255.255.255

! Cho phép ra Internet

permit ip any any

!

interface Vlan30

ip access-group HQ_ISOLATE_IN in

interface Vlan50

ip access-group HQ_ISOLATE_IN in

interface Vlan60

ip access-group HQ_ISOLATE_IN in

PHẦN 2: CẤU HÌNH TẠI CHI NHÁNH BÌNH DƯƠNG (BD)

Áp dụng trên BD-MLS1 và BD-MLS2

Cisco CLI

config terminal

! --- 1. ACL cho Bác sĩ & Y tá (Được về Server HQ) ---

ip access-list extended BD_MEDICAL_IN

permit udp any any eq bootps

permit udp any any eq domain

! QUAN TRỌNG: Cho phép đi xuyên Tunnel về Server HQ

permit ip any 10.10.16.128 0.0.0.63

! Zero Trust: Chặn truy cập nội bộ (VLAN bên cạnh) và chi nhánh khác

deny   ip any 10.0.0.0 0.255.255.255

permit ip any any

!

interface Vlan10

ip access-group BD_MEDICAL_IN in

interface Vlan20

ip access-group BD_MEDICAL_IN in

! --- 2. ACL Cô lập hoàn toàn (Staff, Wifi, Guest) ---

ip access-list extended BD_ISOLATE_IN

permit udp any any eq bootps

permit udp any any eq domain

! Chặn toàn bộ 10.0.0.0/8 (Không về được HQ, không sang được VLAN khác)

deny   ip any 10.0.0.0 0.255.255.255

permit ip any any

!

interface Vlan30

ip access-group BD_ISOLATE_IN in

interface Vlan50

ip access-group BD_ISOLATE_IN in

interface Vlan60

ip access-group BD_ISOLATE_IN in

! --- 3. Bảo vệ VLAN Management ---

ip access-list extended ACL_MGMT_PROTECT

permit udp any any eq 1985

permit ospf any any

! Cho phép Admin PC (ví dụ 10.20.99.10)

permit ip host 10.20.99.10 any

deny   ip any any

!

interface Vlan99

ip access-group ACL_MGMT_PROTECT in

exit

PHẦN 3: CẤU HÌNH TẠI CHI NHÁNH ĐỒNG NAI (DN)

Tương tự Bình Dương, nhưng áp dụng trên DN-MLS1/2

Cisco CLI

config terminal

! --- 1. ACL Y tế (Cho phép về Server HQ) ---

ip access-list extended DN_MEDICAL_IN

permit udp any any eq bootps

permit udp any any eq domain

permit ip any 10.10.16.128 0.0.0.63

deny   ip any 10.0.0.0 0.255.255.255

permit ip any any

!

interface Vlan10

ip access-group DN_MEDICAL_IN in

interface Vlan20

ip access-group DN_MEDICAL_IN in

! --- 2. ACL Cô lập (Staff, Guest) ---

ip access-list extended DN_ISOLATE_IN

permit udp any any eq bootps

permit udp any any eq domain

deny   ip any 10.0.0.0 0.255.255.255

permit ip any any

!

interface Vlan30

ip access-group DN_ISOLATE_IN in

interface Vlan50

ip access-group DN_ISOLATE_IN in

interface Vlan60

ip access-group DN_ISOLATE_IN in

! (Đừng quên bảo vệ VLAN 99 tương tự BD)

exit

PHẦN 4: CẤU HÌNH TẠI CHI NHÁNH CẦN THƠ (CT)

Áp dụng trên CT-MLS1/2

Cisco CLI

config terminal

! --- 1. ACL Y tế ---

ip access-list extended CT_MEDICAL_IN

permit udp any any eq bootps

permit udp any any eq domain

permit ip any 10.10.16.128 0.0.0.63

deny   ip any 10.0.0.0 0.255.255.255

permit ip any any

!

interface Vlan10

ip access-group CT_MEDICAL_IN in

interface Vlan20

ip access-group CT_MEDICAL_IN in

! --- 2. ACL Cô lập ---

ip access-list extended CT_ISOLATE_IN

permit udp any any eq bootps

permit udp any any eq domain

deny   ip any 10.0.0.0 0.255.255.255

permit ip any any

!

interface Vlan30

ip access-group CT_ISOLATE_IN in

interface Vlan50

ip access-group CT_ISOLATE_IN in

interface Vlan60

ip access-group CT_ISOLATE_IN in

exit

## Cấu hình NAT Overload(trên router)

* Chi nhánh BD

+ Branch-BD1

ip route 0.0.0.0 0.0.0.0 100.0.0.9

! Tạo ACL Mở rộng cho NAT

ip access-list extended ACL_NAT_INTERNET

! QUAN TRỌNG: KHÔNG NAT traffic đi đến các mạng nội bộ khác

deny   ip 10.20.0.0 0.0.255.255 10.0.0.0 0.255.255.255

! CHO PHÉP NAT tất cả traffic 10.x.x.x đi ra Internet

permit ip 10.20.0.0 0.0.255.255 any

ip nat inside source list ACL_NAT_INTERNET interface Se0/3/0 overload

int G0/0

ip nat inside

int G0/1

ip nat inside

int se0/3/0

ip nat outside

* Chi nhánh HQ

+ HQ1

config terminal

! Tạo ACL Mở rộng cho NAT

ip access-list extended ACL_NAT_INTERNET

! QUAN TRỌNG: KHÔNG NAT traffic đi đến các mạng nội bộ khác

deny   ip 10.10.0.0 0.0.255.255 10.0.0.0 0.255.255.255

! CHO PHÉP NAT tất cả traffic 10.x.x.x đi ra Internet

permit ip 10.10.0.0 0.0.255.255 any

ip nat inside source list ACL_NAT_INTERNET interface Se0/3/0 overload

int G0/0

ip nat inside

int G0/1

ip nat inside

int se0/3/0

ip nat outside

* Chi nhánh DN

+ Branch-DN1

ip route 0.0.0.0 0.0.0.0 100.0.0.17

! Tạo ACL Mở rộng cho NAT

ip access-list extended ACL_NAT_INTERNET

! QUAN TRỌNG: KHÔNG NAT traffic đi đến các mạng nội bộ khác

deny   ip 10.30.0.0 0.0.255.255 10.0.0.0 0.255.255.255

! CHO PHÉP NAT tất cả traffic 10.x.x.x đi ra Internet

permit ip 10.30.0.0 0.0.255.255 any

ip nat inside source list ACL_NAT_INTERNET interface Se0/3/0 overload

int G0/0

ip nat inside

int G0/1

ip nat inside

int se0/3/0

ip nat outside

* Chi nhánh CT

+ Branch-CT1

ip route 0.0.0.0 0.0.0.0 100.0.0.25

! Tạo ACL Mở rộng cho NAT

ip access-list extended ACL_NAT_INTERNET

! QUAN TRỌNG: KHÔNG NAT traffic đi đến các mạng nội bộ khác

deny   ip 10.40.0.0 0.0.255.255 10.0.0.0 0.255.255.255

! CHO PHÉP NAT tất cả traffic 10.x.x.x đi ra Internet

permit ip 10.40.0.0 0.0.255.255 any

ip nat inside source list ACL_NAT_INTERNET interface Se0/3/0 overload

int G0/0

ip nat inside

int G0/1

ip nat inside

int se0/3/0

ip nat outside

## Yêu cầu "Y tế từ xa (Telemedicine)"

## 1. Cấu hình Chung (Giống hệt nhau cho cả 8 Router)

config terminal ! ! ----- BƯỚC 1: TẠO ACL ĐỂ "NHẬN DIỆN" TRAFFIC Y TẾ ----- ! (Bao gồm tất cả VLAN Bác sĩ và Y tá từ 4 địa điểm) ! ip access-list extended ACL_QOS_TELEMEDICINE remark === Uu tien traffic Y te (Bac si & Y ta) === ! HQ permit ip 10.10.10.0 0.0.1.255 any permit ip 10.10.12.0 0.0.3.255 any ! Binh Duong permit ip 10.20.10.0 0.0.0.255 any permit ip 10.20.20.0 0.0.1.255 any ! Dong Nai permit ip 10.30.10.0 0.0.0.255 any permit ip 10.30.20.0 0.0.1.255 any ! Can Tho permit ip 10.40.10.0 0.0.0.255 any permit ip 10.40.20.0 0.0.1.255 any ! ! ----- BƯỚC 2: TẠO CLASS-MAP ĐỂ PHÂN LOẠI TRAFFIC ----- ! class-map match-all CMAP_TELEMEDICINE match access-group name ACL_QOS_TELEMEDICINE ! ! ----- BƯỚC 3: TẠO POLICY-MAP ĐỂ ƯU TIÊN (HÀNH ĐỘNG) ----- ! policy-map PMAP_WAN_OUT ! ! Class "TELEMEDICINE" được ưu tiên 30% băng thông ! (Lệnh 'priority' đảm bảo độ trễ thấp - Low Latency Queueing) class CMAP_TELEMEDICINE priority percent 30 ! ! Class "class-default" là tất cả traffic còn lại ! (Sẽ chia sẻ 70% băng thông còn lại một cách công bằng) class class-default fair-queue ! end

## 2. Áp dụng Chính sách QoS (Khác nhau cho từng Router)

+ HQ1

config terminal ! Áp dụng vào cổng ra Internet interface Serial0/3/0 service-policy output PMAP_WAN_OUT ! Áp dụng vào các Tunnel interface Tunnel1 service-policy output PMAP_WAN_OUT interface Tunnel2 service-policy output PMAP_WAN_OUT interface Tunnel5 service-policy output PMAP_WAN_OUT interface Tunnel6 service-policy output PMAP_WAN_OUT interface Tunnel9 service-policy output PMAP_WAN_OUT interface Tunnel10 service-policy output PMAP_WAN_OUT ! end

HQ2

config terminal ! interface Serial0/3/0 service-policy output PMAP_WAN_OUT ! interface Tunnel3 service-policy output PMAP_WAN_OUT ! interface Tunnel4 service-policy output PMAP_WAN_OUT ! interface Tunnel7 service-policy output PMAP_WAN_OUT ! interface Tunnel8 service-policy output PMAP_WAN_OUT ! interface Tunnel11 service-policy output PMAP_WAN_OUT ! interface Tunnel12 service-policy output PMAP_WAN_OUT ! end

Branch-BD1

config terminal ! interface Serial0/3/0 service-policy output PMAP_WAN_OUT ! interface Tunnel1 service-policy output PMAP_WAN_OUT ! interface Tunnel3 service-policy output PMAP_WAN_OUT ! end

Branch-BD2

config terminal ! interface Serial0/3/0 service-policy output PMAP_WAN_OUT ! interface Tunnel2 service-policy output PMAP_WAN_OUT ! interface Tunnel4 service-policy output PMAP_WAN_OUT ! end

Branch-DN1

config terminal ! interface Serial0/3/0 service-policy output PMAP_WAN_OUT ! interface Tunnel5 service-policy output PMAP_WAN_OUT ! interface Tunnel7 service-policy output PMAP_WAN_OUT ! end

Branch-DN2

config terminal ! interface Serial0/3/0 service-policy output PMAP_WAN_OUT ! interface Tunnel6 service-policy output PMAP_WAN_OUT ! interface Tunnel8 service-policy output PMAP_WAN_OUT ! end

Branch-CT1

config terminal ! interface Serial0/3/0 service-policy output PMAP_WAN_OUT ! interface Tunnel9 service-policy output PMAP_WAN_OUT ! interface Tunnel11 service-policy output PMAP_WAN_OUT ! end

Branch-CT2

config terminal ! interface Serial0/3/1 service-policy output PMAP_WAN_OUT ! interface Tunnel10 service-policy output PMAP_WAN_OUT ! interface Tunnel12 service-policy output PMAP_WAN_OUT ! end

## Hỗ trợ truy cập từ xa với MFA

+ Trên HQ1 và HQ2

Table 17

config terminal ! ! ----- BƯỚC 1: CẤU HÌNH AAA (ĐỂ SỬ DỤNG RADIUS/MFA) ----- ! (Giả sử bạn có 1 Server RADIUS ở VLAN 40 tên 10.10.16.150) aaa new-model ! radius server RADIUS_MFA_SERVER address 10.10.16.150 auth-port 1812 acct-port 1813 key YourSharedSecretKey ! ! Trong báo cáo, bạn ghi: "RADIUS_MFA_SERVER được kết nối với Azure AD để thực thi MFA" aaa authentication login VPN_AUTH_LIST group RADIUS_MFA_SERVER local aaa authorization network VPN_AUTHOR_LIST group RADIUS_MFA_SERVER local ! ! ----- BƯỚC 2: TẠO DẢI IP ĐỂ CẤP CHO BÁC SĨ Ở NHÀ ----- ip local pool POOL_REMOTE_DOCTORS 10.255.255.1 10.255.255.100 ! ! ----- BƯỚC 3: CẤU HÌNH ISAKMP (PHASE 1) ----- crypto isakmp policy 10 encr aes authentication pre-share group 5 ! ! ----- BƯỚC 4: CẤU HÌNH NHÓM CLIENT (NƠI GẮN AAA) ----- crypto isakmp client configuration group GROUP_REMOTE_DOCTORS key YourClientSharedKey pool POOL_REMOTE_DOCTORS aaa authentication-list VPN_AUTH_LIST aaa authorization-list VPN_AUTHOR_LIST ! ! ----- BƯỚC 5: CẤU HÌNH IPSEC (PHASE 2) ----- crypto ipsec transform-set TSET_REMOTE_VPN esp-aes esp-sha-hmac ! ! ----- BƯỚC 6: TẠO DYNAMIC MAP (VÌ KHÔNG BIẾT IP CỦA BÁC SĨ) ----- crypto dynamic-map DMAP_REMOTE_USERS 10 set transform-set TSET_REMOTE_VPN ! ! ----- BƯỚC 7: TẠO CRYPTO MAP VÀ ÁP DỤNG ----- crypto map CMAP_REMOTE_VPN 10 ipsec-isakmp dynamic DMAP_REMOTE_USERS ! ! Áp dụng vào cổng WAN (nơi nhận kết nối từ Bác sĩ ở nhà) interface Serial0/3/0 crypto map CMAP_REMOTE_VPN ! end

## Cấu hình Cloud & AI Ingretation

+ Trên HQ1 và HQ2

Table 18

config terminal ! ! ----- BƯỚC 1: TẠO TUNNEL VPN MỚI ĐẾN AZURE ----- ! (Giả sử IP Public của Azure VPN Gateway là 20.20.20.20) ! interface Tunnel99 description VPN-TO-AZURE-CLOUD ! (IP Tunnel này do bạn và Azure quy định) ip address 172.16.100.1 255.255.255.252 tunnel source Serial0/3/0 tunnel destination 20.20.20.20 ! (Thêm IPsec protection cho tunnel này) ! ! ----- BƯỚC 2: CẤU HÌNH ĐỊNH TUYẾN (ROUTING) ----- ! (Dạy cho OSPF biết đường đến Azure Cloud, giả sử mạng Azure là 10.100.0.0/16) ! ! (Cách 1: Static route) ip route 10.100.0.0 255.255.0.0 172.16.100.2 ! ! (Cách 2: OSPF - Nếu Azure hỗ trợ BGP/OSPF) router ospf 1 passive-interface Tunnel99 network 172.16.100.0 0.0.0.3 area 0 ! ! ----- BƯỚC 3: CẤU HÌNH ACL (ZERO TRUST) ----- ! (Chỉ cho phép VLAN 40 nói chuyện với Azure) ! ip access-list extended ACL_HQ_TO_AZURE remark === Permit SERVER VLAN to AZURE AI === permit ip 10.10.16.128 0.0.0.63 10.100.0.0 0.0.255.255 remark === Deny all other internal traffic === deny ip 10.0.0.0 0.255.255.255 any ! ! (Cho phép traffic khác ra Internet bình thường) permit ip any any ! ! Áp dụng ACL vào cổng Tunnel đi Azure interface Tunnel99 ip access-group ACL_HQ_TO_AZURE out ! end

Default route

Dựa vào bảng địa chỉ IP kết nối Router - Switch mà bạn cung cấp, đây là cấu hình Default Route (ip route 0.0.0.0 0.0.0.0 ...) cho toàn bộ 8 Switch Layer 3 (MLS).

Mỗi Switch sẽ trỏ Default Gateway về cả 2 Router kết nối trực tiếp với nó. Điều này giúp thiết bị tự động cân bằng tải (Load Balancing) và dự phòng nóng (nếu 1 router chết, router kia sẽ gánh ngay lập tức).

1. Trụ sở chính (TP.HCM - HQ)

Trên HCM-MLS1:

Cisco CLI

config terminal

! Trỏ về Router HQ1

ip route 0.0.0.0 0.0.0.0 10.254.10.1

! Trỏ về Router HQ2

ip route 0.0.0.0 0.0.0.0 10.254.10.9 10

end

Trên HCM-MLS2:

Cisco CLI

config terminal

! Trỏ về Router HQ1

ip route 0.0.0.0 0.0.0.0 10.254.10.5

! Trỏ về Router HQ2

ip route 0.0.0.0 0.0.0.0 10.254.10.13 10

end

2. Chi nhánh Bình Dương (BD)

Trên BD-MLS1:

Cisco CLI

config terminal

! Trỏ về Router BD1

ip route 0.0.0.0 0.0.0.0 10.254.20.1

! Trỏ về Router BD2

ip route 0.0.0.0 0.0.0.0 10.254.20.9 10

end

Trên BD-MLS2:

Cisco CLI

config terminal

! Trỏ về Router BD1

ip route 0.0.0.0 0.0.0.0 10.254.20.5

! Trỏ về Router BD2

ip route 0.0.0.0 0.0.0.0 10.254.20.13 10

end

3. Chi nhánh Đồng Nai (DN)

Trên DN-MLS1:

Cisco CLI

config terminal

! Trỏ về Router DN1

ip route 0.0.0.0 0.0.0.0 10.254.30.1

! Trỏ về Router DN2

ip route 0.0.0.0 0.0.0.0 10.254.30.9 10

end

Trên DN-MLS2:

Cisco CLI

config terminal

! Trỏ về Router DN1

ip route 0.0.0.0 0.0.0.0 10.254.30.5

! Trỏ về Router DN2

ip route 0.0.0.0 0.0.0.0 10.254.30.13 10

end

4. Chi nhánh Cần Thơ (CT)

Trên CT-MLS1:

Cisco CLI

config terminal

! Trỏ về Router CT1

ip route 0.0.0.0 0.0.0.0 10.254.40.1

! Trỏ về Router CT2

ip route 0.0.0.0 0.0.0.0 10.254.40.9 10

end

Trên CT-MLS2:

Cisco CLI

config terminal

! Trỏ về Router CT1

ip route 0.0.0.0 0.0.0.0 10.254.40.5

! Trỏ về Router CT2

ip route 0.0.0.0 0.0.0.0 10.254.40.13 10

end